## Audio Control system

### Equipment Rack

#### 4RU vertical rackmount

#### Rear Access Panel

#### Acceptable product:

SKB Shallow Roto Rack / 1SKB-R4S

### Audio Mixer

#### Four mic inputs with bass cut for vocal microphones and phantom power

#### Two mic/line (XLR/TRS) switchable inputs

#### Two stereo RCA inputs

#### One 1/8" music input

#### Individual input channel metering and output meters

#### Two outputs, XLR and 1/4", each with selectable ducking and limiting functions

#### Fire Alarm interface, Normally Open (NO) or Normally Closed (NC), selectable

#### Acceptable products:

      - Daktronics Multimedia Mixer

### Audio Signal Switch

#### Style: 2 Position

#### Operator action: maintained

#### 1 RU standard rack space, black

#### Laser etched text

#### Acceptable products:

Daktronics Signal Switch

### Laptop Interface

#### 1/8" (3.5 mm) male input

#### Balanced XLR male output

#### Adjustable output volume control

#### Acceptable products:

LTIBLOX Laptop Interface

## Optional Equipment (delete any items that do not apply & All red text)

### USB Audio Interface

#### 24-bit, 96 kHz quality audio

#### Two balanced XLR outputs (left and right)

#### 3.5mm TRS headphone output

#### Ground lift and mono-sum switches

#### Acceptable product:

Radial® Engineering USB-Pro™

## Possible Wired MIC configurations are as follows:

### No Wired Mic Equipment

### Push-to-Talk Mixer + Handheld Vocal Microphone + Dynamic Headphones

### Push-to-Talk Mixer + Single-Muff Headset

### Push-to-Talk Mixer

#### One Balanced Input (XLR) with 24V phantom power

#### One Unbalanced Stereo Input (TRS)

#### One Balanced Output (XLR)

#### 1/4" Headphone Output (TRS)

#### Bluetooth® wireless music input

#### Momentary and continuous audio output modes

#### 4-segment LED metering with Overload (OL) indication

#### Variable bass cut

#### Auto music reduction (ducking)

#### Output limiting

#### Software switchable Line to Mic Pad

#### Individual source level control

#### Acceptable product:

Daktronics SSR-DM2

### Handheld Vocal Microphone

#### Dynamic microphone with unidirectional (cardioid) polar pattern

#### Frequency response: 50 Hz to 15 kHz

#### Acceptable product:

Shure SM58

### Dynamic Headphones

#### Neodymium magnet

#### Gold 1/4" and 1/8" plug

#### Acceptable product:

Sony MDR7506

### Single-Muff Headset

#### Neodymium magnet

#### Cardioid condenser microphone

#### Acceptable product:

Beyerdynamic DT 287

## POSSIBLE WIRELESS configurations are as follows:

### No Wireless Equipment

### Wireless Microphone Receiver + Wireless Handheld Transmitter

### Wireless Microphone Receiver + Wireless Bodypack Transmitter + Referee Headset

### Wireless Microphone Receiver + Additional Wireless Microphone Receiver + Wireless Handheld Transmitter + Additional Wireless Handheld Transmitter

### Wireless Microphone Receiver + Additional Wireless Microphone Receiver + Wireless Handheld Transmitter + Wireless Bodypack Transmitter + Referee Headset

### Any wireless system above may have the High Gain Antenna Kit

### Wireless Microphone Receiver

#### 1/2 RU unit

#### 24 bit digital audio

#### XLR balanced mic/line outputs

#### RF Sensitivity: −97 dBm at 10−5 BER

#### Audio Frequency Response: 20 Hz to 20 kHz, +/- 1 dB

#### Audio Dynamic Range: > 120 dB (A)

#### Total Harmonic Distortion: < 0.1%

#### Acceptable products:

Electro-Voice RE3-RX

### Additional Wireless Microphone Receiver

#### 1/2 RU unit

#### 24 bit digital audio

#### XLR balanced mic/line outputs

#### RF Sensitivity: −97 dBm at 10−5 BER

#### Audio Frequency Response: 20 Hz to 20 kHz, +/- 1 dB

#### Audio Dynamic Range: > 120 dB (A)

#### Total Harmonic Distortion: < 0.1%

#### Acceptable products:

Electro-Voice RE3-RX

### Wireless Handheld Transmitter

#### Microphone element: Cardioid, Dynamic Vocal Microphone

#### Working Range: 100 m (328')

#### Battery life: 9+ hours

#### RF Output: Selectable 1 mW or 10 mW

#### Acceptable products:

Electro-Voice RE3-HHT76

### Additional Wireless Handheld Transmitter

#### Microphone element: Cardioid, Dynamic Vocal Microphone

#### Working Range: 100 m (328')

#### Battery life: 9+ hours

#### RF Output: Selectable 1 mW or 10 mW

#### Acceptable products:

Electro-Voice RE3-HHT76

### Wireless Bodypack Transmitter

#### Connector: 4-pin male mini connector (TA4M) for microphone or mute switch

#### Working Range: 100 m (328')

#### Battery life: 9+ hours

#### RF output: Selectable 1 mW or 10 mW

#### Acceptable products:

Electro-Voice RE3-BPT

### Referee Headset

#### Electret condenser microphone element

#### Unidirectional cardioid polar pattern

#### Acceptable product:

Electro-Voice HM7

### High Gain Antenna Kit

#### Cardioid directional antenna

#### 7 dB gain

#### Acceptable product:

Lectrosonics ALP620 @ 2

### Single-Muff Headset

#### Neodymium magnet

#### Cardioid condenser microphone

#### Acceptable product:

Beyerdynamic DT 287