## Audio Control system

### Equipment Rack

#### Top is (10U) angled rackmount rails + 6" in rear for cables

#### (6U) vertical rackmount space below

#### Locking covers

#### Acceptable products:

SKB 10x6 Roto Rack Console

### Audio Mixer

#### Eight mic/line inputs

#### Two 1/4" stereo inputs

#### Three band EQ per channel

#### Frequency response, mic/line input to any output: 20 Hz to 20 kHz +/- 0.5 dB

#### One insert per channel

#### Gain control per channel: +5 to +60 dB

#### Two aux outputs (pre or post-fader selectable)

#### 2-track inputs and outputs: Unbalanced RCA connector

#### Acceptable products:

Soundcraft EPM8

### Rackmount Surge Suppressor

#### 9 outlets

#### Resettable circuit breaker

#### Max surge current: 50 kA

#### Joule rating: 1800 joules

#### Acceptable product:

Middle Atlantic PD-915R

### Audio Signal Switch

#### Style: 2 Position

#### Operator action: maintained

#### 1 RU standard rack space, black

#### Laser etched text

#### Acceptable products:

Daktronics Signal Switch

### Wireless Microphone Receiver

#### 1/2 RU unit

#### 24 bit digital audio

#### XLR balanced mic/line outputs

#### RF Sensitivity: −97 dBm at 10−5 BER

#### Audio Frequency Response: 20 Hz to 20 kHz, +/- 1 dB

#### Audio Dynamic Range: > 120 dB (A)

#### Total Harmonic Distortion: < 0.1%

#### Acceptable products:

Electro-Voice RE3-RX

### Wireless Handheld Transmitter

#### Microphone element: Cardioid, Dynamic Vocal Microphone

#### Working Range: 100 m (328')

#### Battery life: 9+ hours

#### RF Output: Selectable 1 mW or 10 mW

#### Acceptable products:

Electro-Voice RE3-HHT76

### Professional CD/Media Player

#### MP3, WAV, AAC, and WMA file playback

#### Support for SD/SDHC cards and USB memory devices

#### Wireless playback via BluetoothTM devices

#### AM/FM radio tuner

#### Balanced XLR and Unbalanced RCA outputs

#### Acceptable products:

TASCAM CD-400U

Denon DN-300Z

### Laptop Interface

#### 1/8" (3.5 mm) male input

#### Balanced XLR male output

#### Adjustable output volume control

#### Acceptable products:

LTIBLOX Laptop Interface

## Optional Equipment (delete any items that do not apply & all red text)

### USB Audio Interface

#### 24-bit, 96 kHz quality audio

#### Two balanced XLR outputs (left and right)

#### 3.5mm TRS headphone output

#### Ground lift and mono-sum switches

#### Acceptable product:

Radial® Engineering USB-Pro™

### Self-powered Monitor Speaker

#### Three inputs: one 1/4" phone, one RCA, one XLR

#### Tone control (High/Low)

#### Acceptable product:

Yamaha MSP3

## Possible Wired MIC configurations are as follows:

### No Wired Mic Equipment

### Push-to-Talk Mixer + Handheld Vocal Microphone + Dynamic Headphones

### Push-to-Talk Mixer + Single-Muff Headset

### Push-to-Talk Mixer

#### One Balanced Input (XLR) with 24V phantom power

#### One Unbalanced Stereo Input (TRS)

#### One Balanced Output (XLR)

#### 1/4" Headphone Output (TRS)

#### Bluetooth® wireless music input

#### Momentary and continuous audio output modes

#### 4-segment LED metering with Overload (OL) indication

#### Variable bass cut

#### Auto music reduction (ducking)

#### Output limiting

#### Software switchable Line to Mic Pad

#### Individual source level control

#### Acceptable product:

Daktronics SSR-DM2

### Handheld Vocal Microphone

#### Dynamic microphone with unidirectional (cardioid) polar pattern

#### Frequency response: 50 Hz to 15 kHz

#### Acceptable product:

Shure SM58

### Dynamic Headphones

#### Neodymium magnet

#### Gold 1/4" and 1/8" plug

#### Acceptable product:

Sony MDR7506

### Single-Muff Headset

#### Neodymium magnet

#### Cardioid condenser microphone

#### Acceptable product:

Beyerdynamic DT 287

## POSSIBLE WIRLESS configurations are as follows:

### Additional Wireless Microphone Receiver + Wireless Bodypack Transmitter

### Additional Wireless Microphone Receiver + Wireless Bodypack Transmitter + Referee Headset + In-Ear Monitor System + ADA Hearing Assist

### Any wireless system above may have the High Gain Antenna Kit

### Additional Wireless Microphone Receiver

#### 1/2 RU unit

#### 24 bit digital audio

#### XLR balanced mic/line outputs

#### RF Sensitivity: −97 dBm at 10−5 BER

#### Audio Frequency Response: 20 Hz to 20 kHz, +/- 1 dB

#### Audio Dynamic Range: > 120 dB (A)

#### Total Harmonic Distortion: < 0.1%

#### Acceptable products:

Electro-Voice RE3-RX

### Wireless Bodypack Transmitter

#### Connector: 4-pin male mini connector (TA4M) for microphone or mute switch

#### Working Range: 100 m (328')

#### Battery life: 9+ hours

#### RF output: Selectable 1 mW or 10 mW

#### Acceptable products:

Electro-Voice RE3-BPT

### Referee Headset

#### Electret condenser microphone element

#### Unidirectional cardioid polar pattern

#### Acceptable product:

Electro-Voice HM7

### In-Ear Monitor System

#### Audio frequency response: 25 Hz to 15 kHz

#### Operating range: 300 ft

#### Total harmonic distortion: 0.9%

#### Signal-to-noise ratio: >90 dB

#### RF output power: 50 mW typical

#### Acceptable products:

Sennheiser EW IEM G4

### ADA Hearing Assist

#### Signal-to noise ratio: SQ enabled: 70 dB; SQ disabled 50 dB

#### 57 channels

#### Frequency response: 50 Hz to 15 kHz +/- 3dB

#### Acceptable products:

Listen LS-54-216

### High Gain Antenna Kit

#### Cardioid directional antenna

#### 7 dB gain

#### Acceptable product:

Lectrosonics ALP620 @ 2