SECTION 11 68 43
DAKTRONICS TI-2035 TIMING DISPLAY

Part 1 GENERAL
1.01 SECTION INCLUDES
A. Single-sided LED timing display
1.02 references
A. Standard for Electric Signs, UL 48

B. Standard for CSA C22.2 #207 
C. Federal Communications Commission Regulation Part 15

D. National Electric Code

1.03 SUBMITTALS

A. Product data: Submit manufacturer’s product illustrations, data and literature that fully describe the displays and accessories proposed for installation.


B. Shop drawings: Submit mechanical and electrical drawings.

C. Maintenance data: Submit manufacturer’s installation, operation, and maintenance manuals.

1.04 DELIVERY, STORAGE, AND HANDLING

A. Product delivered on site
B. Display and equipment to be housed in a clean, dry environment
1.05 Project CONDITIONS

A. Environmental limitations: Do not install display equipment until mounting structure is secure and concrete has ample time to cure.
B. Field measurements: Verify position and elevation of structure and its layout for display equipment. Verify dimensions by field measurements.
C. Verify mounting structure is capable of supporting the display’s weight and windload in addition to the auxiliary equipment.
D. Installation may proceed within acceptable weather conditions.

1.06 quality assurance

A. For outdoor use

B. Source Limitations: Obtain each type of scoring or related equipment through one source from a single manufacturer.

C. ETL listed to UL 48 

D. NEC compliant

E. FCC compliant

F. ETLC listed to CSA 22.2 #207

1.07 WARRANTY

A. Provide 1 year of no cost parts exchange including standard shipping on electronics parts and radios due to manufacturing defects
B. Provide toll-free service coordination

C. Provide technical phone support during Daktronics business hours

Part 2 PRODUCTS
2.01 manufacturer
A. Daktronics, Inc., 201 Daktronics Drive, P.O. Box 5128, Brookings, SD 57006-5128
B. ________________________________________________________________________

C. ________________________________________________________________________

2.02 Product

A. Daktronics TI-2035 outdoor LED timer displays game time to 99:59 and a play clock that can count up to or down from any preset time between 0 and 99 seconds. During the last minute of the period, game time is displayed to 1/10 of a second.
2.03 display
A. General information

1. Dimensions: 3'-9" (1.14 m) high, 3'-6" (1.07 m) wide, 0'-8" (203 mm) deep

2. Weight: 55 lb (25 kg)

3. Power requirement: 70 W (red/amber digits), 95 W (red/white digits)
4. Color: provide over 150 colors to choose from
B. Construction
1. Alcoa aluminum alloy 5052 for excellent corrosion resistance
2. Display back, face, and perimeter: 0.063" (1.60 mm) thick 
C. Digits

1. LED color (SELECT ONE DIGIT COLOR OPTION IN BRACKETS ONLY): 
[Red play clock with Amber game time] [Red play clock with White game time]
2. Game time digits: 24" (610 mm) high

3. Play clock digits: 10" (254 mm) high

4. Seven bar segments per digit 
5. PanaView® LED digit technology

6. All digits are sealed front and back with weather-tight silicone gel 
D. Border Striping
1. Vinyl striping applied around the display face 

2. Color: standard white or choose from manufacturer’s vinyl color selections

E. Accessory Equipment 
(SELECT ONE ACCESSORY IN BRACKETS PER NUMBER OR DELETE ENTIRE NUMBER)
1. [Horn – for segment timing operations only]

2. [Carrying handles for portability]

3. [Four-wheel cart for portability]

4. [Power cord and signal jack for portability]

5. [Individual digit protective screens]
2.04 SCORING CONSOLE
A. Console is an All Sport® 5000 controller
B. Scores multiple sports using changeable keyboard inserts

C. Controls multiple displays, including other All Sport 5000 controlled displays currently owned by customer

D. Recalls clock information if power is lost

E. Runs Time of Day and Segment Timer modes

F. Console includes:

1. Rugged aluminum enclosure to house electronics   

2. Sealed membrane water-resistant keyboard   

3. 32-character LCD to verify entries and recall information currently displayed 

4. Power cord that plugs into a standard grounded outlet; 6 watts max

5. Control cable to connect to the control receptacle junction box (wired system only)

6. Hand-held switch for main clock start/stop and horn

7. Soft-sided carrying case
G. Accessory Equipment 
(SELECT ONE ACCESSORY IN BRACKETS PER NUMBER OR DELETE ENTIRE NUMBER)

1. [2.4 GHz spread spectrum radio system with frequency hopping technology and 64 non-interfering channels; system includes a transmitter installed inside the console and a receiver installed inside the display(s)]
2. [Hard carrying case]

3. [Battery pack]

Part 3 EXECUTION

3.01 EXAMINATION

A. Verify that mounting structure is ready to receive display. Verify that placement of conduit and junction boxes are as specified and indicated in plans and shop drawings. Verify concrete has cured adequately according to specifications.

3.02 INSTALLATION

A. All power and control cables to display will be routed in conduit. Power to the display as well as raceways shown on electrical plans by the Electrical Contractor. Display control wiring including conduit will be the responsibility of the contractor assigned the display equipment.
B. Install display to beams in location detailed and in accordance with manufacturer's instructions. Verify unit is plumb and level.

3.03 installation—control center

A. Provide boxes, cover plates and jacks in locations per plans. 

B. Test connect control unit to all jacks and check for proper operation of control unit, display, and all features. Leave control unit in carrying case and other loose accessories with owner's designated representative.
C. Verify earth ground does not exceed 15 ohms.

END OF SECTION

PAGE  
DD3632303  072417
TI-2035
11 68 43-3

