# SECTION 274116.63

# SOUND REINFORCEMENT SYSTEM

# GENERAL

## Summary

### This section includes all professional services, transportation services, supervision, coordination, etc., necessary to complete the installation of a high quality Audio System as specified. “Audio System” references the system and “Audio System Installer” references the installer. The Audio System Installer is responsible for sizing the speaker cluster, amplifiers, patch panels, transformers, etc. to ensure a complete and properly operating system in accordance with the performance criteria set forth in this specification. All installed equipment must be new and in unused condition. It is the intent of the specifications to indicate the quality, configuration, and performance of the Audio System. The Audio System installation includes the following:

#### Point Source Sound System

#### Audio Control System

#### Cables, connectors, plates and wiring

#### All necessary design, fabrication, processing and amplification equipment, and installation for a complete sound system as described

### The section also includes:

#### Scope of work

#### Verification of dimensions and conditions at the job site

#### Preparation of submittal information

#### Installation in accordance with the contract documents, manufacturers recommendations, and all applicable code requirements

#### Instruction of operating personnel: provision of manuals

#### Maintenance services and warranty

## REFERENCES

### Audio System Design and Installation, G.H. Philip Giddings, 1990

### Digital Audio Engineering - Serial Transmission Format for Two-Channel Linearly Represented Digital Audio Data, Recommended Practice (AES-3), ANSI S4.40, 1992

### Handbook for Sound Engineers (Third Edition) Glen M. Ballou, 2002

### Loudspeaker Components Used in Professional Audio and Sound Reinforcement, ANSI S4.26, 1984 (R1992)

### Sound System Engineering (Third Edition), Don Davis and Eugene Patronis, Jr., 2006

### Standard for Safety for Audio, Video, and Similar Electronic Apparatus – Safety Requirements: IEC 60065, UL 60065, EN 60065, and CAN/CSA C22.2 #60065

### Information Technology Equipment – Radio Disturbance Characteristics: EN 55022:2010 / AC:2011, Class A / CISPR 22 (ed.5); am1

### Electromagnetic Compatibility: EN 61000-3-2:2006 +A1:2009 +A2:2009 / IEC 61000-3-2 (ed.3); am1, am2

### Electromagnetic Compatibility: EN 61000-3-3:2008 / IEC 61000-3-3 (ed.2)

### Information Technology Equipment – Immunity Characteristics: EN 55024:2010 / CISPR 24 (ed.1); am1, am2;  47 CFR, Part 15:2010, §15.107 and §15.109, Class A;  ICES-003, Issue 4:2004

### National Electric Code

## SYSTEM DESCRIPTION

### The following is intended to provide an overview of the design concepts and is not an exhaustive description of the Audio System.

### The Audio System shall consist of a main speaker cluster located above center court. 
The Audio System shall provide full range sound reinforcement capable of high speech intelligibility and quality music reproduction. The speaker components shall be of the two- or three-way type, capable of providing full range sound coverage. The speaker components shall be adjustable to cover each seating area independently.

### Amplification and related signal processing equipment shall be housed in a new steel equipment rack to be located at the current amplifier rack location. Power amplifiers shall be sized to provide 20% headroom above the maximum power handling rating for each speaker component in the system. The minimum load impedance of each amplifier channel shall not be below 3 ohms.

### Scope of work includes the following: Physical mounting of main speaker cluster to the roof truss, speaker cable between the speaker cluster and wall mount rack supplied by the Audio System Installer, signal cable between the control location (designated by owner) and the wall mount rack, a dedicated AC power feed (supplied by owner) to the supplied wall mount rack, and the audio control system. A licensed electrician must perform all high voltage electrical work.

### Loudspeaker

#### Safe, secure and permanent manner in their operating position.

#### The process of performance testing the system may necessitate moving and adjusting the loudspeakers. Provide for 0-25 degrees of vertical adjustability down in 5 degree increments for the Main Seating loudspeakers. Provide for 0-90 degrees vertical adjustability down for the Court Coverage loudspeakers. Perform such adjustments without claim for additional payment.

#### Rigging, mounting and support systems for loudspeakers reviewed and certified by a registered professional engineer.

#### Structural support members to have an appropriate safety factor (determined by a structural engineer). All fasteners to be graded and certified for use in the intended applications.

### The Audio System is to achieve the following performance standards: continuous output level of 120 dBA at 1 meter with sufficient headroom to allow for peaks. The frequency response of the sound system to be at least 35 Hz to 20 kHz  (-10 dB) at 1 meter.

### The Audio System utilizes digital signal processing for all applications. All digital signal processing is contained within the amplifier. The digital signal processor will provide the following signal processing capability: output gain control, equalization, delay, crossover, and limiting.

## SUBMITTALS

### Submit the following documents with bid proposal. Failure to provide a complete set of submittals will result in disqualification. Provide shop drawings and submittal data containing sufficient information to describe the work to be performed. Prepare drawings at an appropriate scale. Submit shop drawing information at one time. Provide the following information, but not necessarily limited to:

#### System description

#### Complete system equipment list, with individual specification sheets for each piece of equipment

#### Functional system block diagram showing all major equipment and signal flow

#### Basic design drawings consisting of the speaker cluster’s overall dimensions, and estimated weight of entire system with all equipment installed

#### Proof of performance illustrations as generated by an acoustical CAD modeling program (EASE or equivalent). Illustrations to include direct field sound pressure level performance throughout the facility at 125 Hz, 250 Hz, 500 Hz, 1,000 Hz, 2,000 Hz and 4,000 Hz.

## DELIVERY, STORAGE AND HANDLING

### The Audio System Installer will be responsible for transporting all related audio equipment to the job site. Upon installation, audio equipment becomes the responsibility of the owner.

### The Audio System Installer will be responsible for audio equipment once it is received at the job site. In addition, Audio System Installer will be responsible for unloading the shipment truck, providing safe storage of audio equipment, and installation of audio equipment.

### All transportation expenses of equipment to the job site will be the responsibility of the Audio System Installer.

## PROJECT CONDITIONS

### Audio System Installer desiring to submit a proposal are responsible for acquiring, from the owner, any plans or documentation pertaining to the audio system installation and are encouraged to perform a pre-bid site survey.

### Confirm conditions on the jobsite pertinent to this work. Give notice to the owner in writing of discrepancies, conflict, or omissions promptly upon discovery.

## QUALITY ASSURANCE

### Source Limitations: Obtain each type of audio equipment through one source from a single manufacturer.

### Manufacturer’s qualifications: A minimum of five (5) years’ experience with the specified types of products and installation.

### Installer’s qualification: Business familiar in the installation of systems similar in complexity to those essential for this project; and fulfillment of the following:

#### The primary business of contractor shall be in the installation of sound and video systems.

#### At least (5) five years’ experience with systems of the specified types and products included.

#### Experience with comparable scale sound reinforcement projects within the last three (3) years.

#### Retain a fully staffed and equipped service facility with fulltime field technicians.

#### Be a franchised dealer and approved service facility for all amplifiers, digital and analog signal processing equipment and loudspeaker products specified or proposed; if not, supply detailed description of how warranty service on these items will be obtained, and if any manufacturer’s warranties will become void.

#### Installer to be factory educated in the installation and maintenance of any digital signal processed based control systems.

#### At the request of the owner, the Installer must demonstrate having:

Sufficient plant and equipment to complete the work within the agreed timetable

Sufficient staff with commensurate technical experience

Appropriate financial status to meet the obligations of the work

Capability to provide performance bonding

## WARRANTY

### Provide a complete system parts warranty for a minimum period of one year after completion and acceptance of audio system.

### This warranty will not void specific warranties issued by manufacturers for greater periods of time. Nor will it void any rights guaranteed to the owner by law.

### The Audio System Installer must provide the owner an opportunity to purchase a preventive maintenance inspection agreement or full service agreement. The minimum term of any maintenance or service agreement consists of one year.

### Warranty replacement and service of equipment will not apply to any owner-furnished equipment.

# PRODUCTS

## Acceptable Manufacturers

### To establish the minimum functional, aesthetic and quality standards, products and product series of the manufacturers listed below are required.

### Substitutions: Products of other manufacturers are acceptable provided they meet the performance and reliability standards of the recommended equipment. Any substitution to this list must be submitted and approved in writing 10 days prior to the bid date.

## General

### Regardless of the length or completeness of the descriptions below, each device shall meet published manufacturers’ specifications.

### Equipment and materials must be new and conform to applicable UL or ANSI provision.

### Product quantity is as required. If a quantity is given, Audio System Installer shall provide at least the given amount.

## Main Speaker Cluster

### Engineered single source speaker system

#### Frequency response (-10 dB @ 1 m): 45 Hz to 20 kHz

#### Max SPL (@ 1 m): 120 dBA

#### Overall coverage: Main Seating = 45 V X 150 H (@ 2), Court Coverage = 85 V x 110 H (@ 2)

#### Acceptable products:

Daktronics Sportsound® SSN-250 System (@ 1)

JBL CBT 70J-1 + 70JE-1 Array System (@ 2), Control 29AV-1 (@ 2), and ASB6112 (@ 1)

## Control Rack

### Rack

#### Pivoting, sectional wall cabinet with 12RU of rack space

#### Tamper resistant design ideal for use in non-secure areas

#### Vented design for effective passive thermal management

#### Welded steel construction with 200 lb weight capacity

#### 3RU fully-enclosed drawer with customizable foam insert for secure storage

#### Acceptable products:

Middle Atlantic DWR-12-26 (@ 1) and UD3 (@ 1)

### Amplifier

#### 4 Channel; 700 W per channel

#### Frequency response: 20 Hz to 20 kHz +/-0.5 dB

#### Signal to noise: 105 dB (20 Hz to 20 kHz referenced to 8 Ω)

#### THD+N: 0.1% (20 Hz to 20 kHz)

#### On-board Digital Signal Processing (DSP)

#### Acceptable products:

      - LEA Connect Series 704 @ 1

### Rackmount Surge Suppressor

#### Maximum AC Current Rating: 20AMPS

#### Extreme Voltage Shutdown Protection

#### Sequencing capabilities with 3 separate delays

#### Remote power switching capabilities

#### Fire alarm shutoff capable

#### 1RU unit

#### Acceptable product:

Furman CN2400S

## Optional Equipment (delete any items that do not apply & All red text)

### Remote System Control Panel

#### Locking Key Switch

#### “System On” Indicator LED

#### Mounts to 1-gang junction box

#### Acceptable product:

Furman RS1

## audio Control system (Copy & Paste the text from one of the appropriate documents linked below in place of this section)

### SSR-100 – DD2750076

### SSR-200 – DD2750078

## Cable and Control Wiring

### All cables must be installed in conduit or closed raceway areas. Use plenum cable as necessary. Exposed cable is not acceptable, unless approved by owner. Cable specifications are as follows:

#### 8 conductor, 13 AWG speaker cable (individually colored) from speaker cluster to amplifier rack location

#### 1 pair, 22 AWG signal cable from amplifier rack to input plate(s)

#### Microphone level cables: No. 22 shielded jacketed - Belden 9451 with black jacket

# EXECUTION

## general

### Coordinate work with other trades to prevent delays in the construction schedule.

### Verify dimensions and location of equipment to be mounted.

## Installation

### Mount speaker cluster and wall rack square and plumb. Permanently installed equipment to be held firmly and safely in place. All mounting brackets must be approved by a structural engineer.

### Input Plates shall be 1/8" thick aluminum, standard EIA sizes, brushed black anodized finish unless otherwise noted. Plastic plates are not acceptable. A maximum of 4 inputs may be distributed in venue. It is the Installer’s responsibility to determine the exact quantity and location of microphone input plates needed. Verify with the owner.

### Remote mount antennas from control rack location to venue for clear line of sight with transmitters.

## system wiring

### For all analog audio interfaces, take precautions to prevent and guard against electromagnetic and electrostatic hum. For analog line level audio signals, float cable shield at the receiver end of the cable. Shields not connected to be folded back over cable jacket and covered with heat-shrink tubing. Do not cut off unused shields.

### Exercise care in wiring; damaged cables or equipment are not acceptable. Isolate cables of different signals or different levels; and separate, organize, and route to restrict channel crosstalk or feedback oscillation in any amplifier section. Keep wiring separated into groups for microphone level circuits, loudspeaker circuits, and power circuits.

### Make joints and connections with rosin-core solder or with audio grade mechanical connectors; where spade lugs are used, crimp properly with ratchet type tool. Spade lugs mounted on 22 gauge or smaller cable to be soldered after crimping.

### Microphone, line, and control wiring from receptacle plate/chassis to patch panel/rack to be unbroken and unspliced unless specifically indicated on drawings. Remove spliced cables and replace without additional charge to the owner.

### Connect cable to active components through screw terminal connections and spade lugs whenever available. Make connections to speaker transformers with properly sized closed end connectors crimped with factory approved ratchet type tool. Wire nut or Scotchlock connectors are not acceptable. Do not wrap audio cable splices or connections with adhesive backed tape.

### Execute wiring in strict adherence to professional installation standards, as excerpted from Audio System Design and Installation P. Giddings (1st Edition), Howard W. Sams, 1990, and Appendix II "Recommended Wiring Practices", Sound System Engineering. (3rd Edition), D. Davis.

### Wiring in rack to be neatly tied in manageable bundles with cable lengths cut to minimize excess cable slack but still allow for service and testing. Provide horizontal support bars if cable bundles sag. Eliminate excess AC power cable from rack mounted equipment. Rack wiring to be handled with plastic cable ties or lacing twine; electrical tape and adhesive backed cable tie anchors are not acceptable.

### Connect loudspeakers electrically in phase, using the same wire color code for speaker wiring throughout the project.

### Wiring and connections shall be completely visible and labeled in rack.

## Equipment and Cable Labeling

### Provide printed labels on the front and rear of active equipment mounted in racks. Mount labels in a neat, plumb and permanent manner.

### On multiple input devices such as mixers, provide a printed label over each control that describes the function or purpose of the control. Label size to be adjusted to fit available space.

### Equipment labels to have 1/8" high characters minimum. Labels to be black with white characters, except where indicated.

### Cables and wiring to be logically, legibly, and permanently labeled for easy identification. Labels on cables shall be adhesive strip type. Self-laminating type labels are acceptable. Hand-written labels are not acceptable.

## FIELD QUALITY CONTROL

### Upon completion of installation and initial test adjustments, the audio system installer will conduct a performance evaluation in the presence of the owner or the owner’s representative (owner’s option). The audio system installer will notify the owner or the installer’s representative of the testing schedule.

# END OF SECTION