## Audio Control system

### Equipment Rack

#### Pivoting, sectional wall cabinet with 16RU of rack space

#### Tamper resistant design ideal for use in non-secure areas

#### Vented design for effective passive thermal management

#### Welded steel construction with 250 lb weight capacity

#### Acceptable products:

Middle Atlantic MMR-1620

### Digital Audio Mixer

#### 16x16 processing matrix

#### 8x8 local analog I/O

#### I/O Port for expansion or audio networking, up to 128x128

#### Dante 96 kHz optional cards (AES67 and DDM ready)

#### 16 configurable processing outputs – up to 16 mono / 8 stereo zones

#### Sound management tools:

##### Priority ducking

##### 8-band PEQ, dynamics and delay on every input and zone

##### Processing with x-over filter, delay, limiter and PEQ

#### Acceptable products:

Allen & Heath AHM-16

### Professional CD/Media Player

#### MP3, WAV, AAC, and WMA file playback

#### Support for SD/SDHC cards and USB memory devices

#### Wireless playback via Bluetooth® devices

#### Balanced XLR and Unbalanced RCA outputs

#### Acceptable products:

TASCAM CD-400U

### Rackmount Surge Suppressor

#### 9 outlets

#### Resettable circuit breaker

#### Max surge current: 50 kA

#### Joule rating: 1800 joules

#### Acceptable product:

Middle Atlantic PD-915R

### Audio Signal Switch

#### Style: 2 Position

#### 3 USB jacks to power wireless devices

#### 1 RU standard rack space, black

#### Laser etched text

#### Acceptable products:

Daktronics Signal Switch

### Media Converter/Network Switch

#### 8 Gigabit Ethernet RJ-45 Copper ports (10M/100M/1G) - 1000BASE-T

#### 8 PoE/PoE+ ports

#### 2 Gigabit SFP (fiber) ports (100M/1G) - 1000BASE-X

#### Managed switch with QOS and IGMP snooping

#### Acceptable products:

NETGEAR GS110TP

### Network Router

#### 4-Port Gigabit LAN

#### 1-Port Gigabit WAN

#### Acceptable products:

Mikrotik hEX S

### Remote User Station for Mixer Control

#### Tablet

#### Dual Band Wi-Fi Wireless Access Point (2.4/5.8 GHz)

### Wireless Microphone Receivers

#### 1/2 RU unit

#### 24-bit digital audio

#### XLR balanced mic/line outputs

#### RF Sensitivity: −97 dBm at 10−5 BER

#### Audio Frequency Response: 20 Hz to 20 kHz, +/- 1 dB

#### Audio Dynamic Range: > 120 dB (A)

#### Total Harmonic Distortion: < 0.1%

#### Acceptable products:

Shure QLXD4 @ 2

### Laptop Interface

#### 1/8" (3.5 mm) male input

#### Balanced XLR male output

#### Adjustable output volume control

#### Acceptable products:

LTIBLOX Laptop Interface

## POSSIBLE WIRELESS configurations are as follows:

### Wireless Handheld Transmitters @2 (INDOOR ONLY)

### Wireless Handheld Transmitter @1 + Wireless Bodypack Transmitter @1 + Referee Headset @1 (OUTDOOR ONLY)

### Wireless Handheld Transmitter

#### Microphone element: Cardioid, Dynamic Vocal Microphone

#### Working Range: 100 m (328')

#### Battery life: 9+ hours

#### RF Output: Selectable 1 mW or 10 mW

#### Acceptable products:

Shure QLXD2/58

### Wireless Bodypack Transmitter

#### Connector: 4-pin male mini connector (TA4M) for microphone or mute switch

#### Working Range: 100 m (328')

#### Battery life: 9+ hours

#### RF output: Selectable 1 mW or 10 mW

#### Acceptable products:

Shure QLXD1

### Referee Headset

#### Electret condenser microphone element

#### Unidirectional cardioid polar pattern

#### Acceptable product:

Shure SM35

#### INDOOR AUDIO SYSTEM EQUIPMENT ONLY – DELETE IF RACK WILL BE USED FOR OUTDOOR SYSTEMS

### Amplifier

#### 4 Channel; 700 W per channel

#### Frequency response: 20 Hz to 20 kHz +/-0.5 dB

#### Signal to noise: 105 dB (20 Hz to 20 kHz referenced to 8 Ω)

#### THD+N: 0.1% (20 Hz to 20 kHz)

#### On-board Digital Signal Processing (DSP)

#### Acceptable products:

LEA Connect Series 704 @ 1

## Optional Equipment (delete any items that do not apply & all red text)

### Push-to-Talk Mixer

#### One Balanced Input (XLR) with 24V phantom power

#### One Unbalanced Stereo Input (TRS)

#### One Balanced Output (XLR)

#### 1/4" Headphone Output (TRS)

#### Bluetooth® wireless music input

#### Momentary and continuous audio output modes

#### 4-segment LED metering with Overload (OL) indication

#### Variable bass cut

#### Auto music reduction (ducking)

#### Output limiting

#### Software switchable Line to Mic Pad

#### Individual source level control

#### Acceptable product:

Daktronics SSR-DM2

### High Gain Antenna Kit (for Wireless Microphone Systems)

#### Wideband directional antenna

#### 6 dBi gain

#### Acceptable product:

Wisycom LBN2 @ 2

### In-Ear Monitor System

#### Audio frequency response: 38 Hz to 15 kHz

#### Operating range: 300 ft

#### Total harmonic distortion: <0.5% (typical)

#### Signal-to-noise ratio: 90 dB typical

#### RF output power: 30 mW

#### Acceptable products:

Shure PSM300

### High Gain Antenna Kit (for In-Ear Monitor Systems)

#### Wideband directional antenna

#### 6 dBi gain

#### Acceptable product:

Wisycom LBN2 @ 1

### ADA Hearing Assist

#### Signal-to noise ratio – SQ enabled: 70 dB; SQ disabled 50 dB

#### 57 channels

#### Frequency response: 50 Hz to 15 kHz +/- 3dB

#### Acceptable products:

Listen LS-54-216

### Handheld Vocal Microphone

#### Dynamic microphone with unidirectional (cardioid) polar pattern

#### Frequency response: 50 Hz to 15 kHz

#### Acceptable product:

Shure SM58

### Dynamic Headphones

#### Neodymium magnet

#### Gold 1/4" and 1/8" plug

#### Acceptable product:

Sony MDR7506

### Single-Muff Headset

#### Neodymium magnet

#### Cardioid condenser microphone

#### Acceptable product:

Beyerdynamic DT 287

### Self-powered Monitor Speaker

#### Three inputs: one 1/4" phone, one RCA, one XLR

#### Tone control (High/Low)

#### Acceptable product:

Yamaha MSP3A