SECTION 11 66 43
DAKTRONICS H-2105 SCOREBOARD


Part 1 GENERAL
1.01 SECTION INCLUDES
A.  Four-sided, center hung LED hockey scoreboard
1.02 references

A. Standard for Electric Signs, UL 48

B. Standard for CSA C22.2 #207 
C. Federal Communications Commission Regulation Part 15

D. National Electric Code

1.03 SUBMITTALS

A. Product data: Submit manufacturer’s product illustrations, data and literature that fully describe the scoreboards and accessories proposed for installation.
B. Shop drawings: Submit mechanical and electrical drawings.

C. Maintenance data: Submit manufacturer’s installation, operation, and maintenance manuals.

1.04 DELIVERY, STORAGE, AND HANDLING

A. Product delivered on site
B. Scoreboard and equipment to be housed in a clean, dry environment
1.05 Project CONDITIONS

A. Environmental Limitations: Do not install scoring equipment until spaces are enclosed and weatherproof, wet work in spaces is complete and dry, and ambient temperature and humidity conditions are maintained at the levels indicated for project when occupied for its intended use.

B. Field Measurements: Coordinate scoreboard location and height with the customer. Verify dimensions by field measurements.

C. Supply weight and mounting method for owner to verify that building structure is capable of supporting the scoreboard’s weight in addition to the auxiliary equipment.

1.06 quality assurance
A. For indoor use only
B. Source Limitations: Obtain each type of scoring equipment and electronic displays through one source from a single manufacturer.

C. ETL listed to UL 48 

D. NEC compliant

E. FCC compliant

F. ETLC listed to CSA 22.2 #207

1.07 WARRANTY
A. Provide 5 years of no cost parts exchange including standard shipping on electronics parts and radios due to manufacturing defects
B. Provide toll-free service coordination

C. Provide technical online and phone support during Daktronics business hours
Part 2 PRODUCTS

2.01 manufacturer

A. Daktronics, Inc., 201 Daktronics Drive, P.O. Box 5128, Brookings, SD 57006-5128

B. ________________________________________________________________________

C. ________________________________________________________________________

2.02 Product

A. Daktronics H-2105 four-sided hockey scoreboard displays period time to 99:59, HOME and GUEST scores to 99, PERIOD to nine and SHOTS ON GOAL to 99 along with PLYR (player number) to 99 and PENALTY time to 9:59 for up to two players on each team. During the last minute of the period, scoreboard displays time to 1/10 of a second. Scoreboard can also score any sport requiring a clock, score and period function.

2.03 scoreboard

A. General information

1. Dimensions: 5'-2" (1.57 m) high, 12'-10" (3.91 m) wide, 12'-10" (3.91 m) deep

2. Base weight: 1100 lb (500 kg) – options may increase weight
3. Base power requirement: 780 W – options may increase wattage
4. Color: choose from manufacturer’s standard paint selections
B. Construction 

1. All-aluminum construction

2. Scoreboard back, face, and perimeter: 0.063" (1.60 mm) thick

3. Cabinet withstands high-velocity impact from air-filled sports balls without the need for protective screens

C. Digits 
1. LED color (SELECT ONE DIGIT COLOR IN BRACKETS ONLY): 
[Amber clock/colon, PERIOD, PLYR and SHOTS ON GOAL digits with Red score and PENALTY digits] 
*or*

[All White digits and indicators]
2. Clock and score digits: 10" (254 mm) high

3. PERIOD, PLYR, PENALTY and SHOTS ON GOAL digits: 7" (178 mm) high

4. Seven bar segments per digit

5. PanaView® (PV) digit technology – discrete LEDs protrude through the scoreboard face
D. Captions

1. Vinyl applied directly to scoreboard face

2. HOME and GUEST captions: 6" (152 mm) high

3. PERIOD, PLYR, PENALTY and SHOTS ON GOAL captions: 4" (102 mm) high
4. Color: standard white or choose from manufacturer’s vinyl color selections
E. Border Striping
1. Vinyl striping applied around the clock and scoreboard face
2. Color: standard white or choose from manufacturer’s vinyl color selections

F. Horns
1. Vibrating horns mounted inside the scoreboard cabinets behind the face
2. Sounds automatically when period clock counts down to zero

3. Sounds manually as directed by operator

G. Power Cords
1. Cords are 11' (3.35 m) long

2. Cords plug into a standard grounded outlet

H. Accessory Equipment 
(SELECT ONE ACCESSORY IN BRACKETS PER NUMBER OR DELETE ENTIRE NUMBER)

1. [Custom vinyl team name captions in place of HOME] 
*or*

[Two 6" (152 mm) high Programmable Team Name Message Centers (TNMCs) in place of vinyl HOME and GUEST captions – add 60 lb (27 kg) and 240 W]

2. [12" (305 mm) high, 36" (914 mm) wide aluminum panel with vinyl logo/sponsor decoration]
3. [Four logo/sponsor areas on corner shrouds between scoreboard faces]

4. [10' (3.05 m) x 10' (3.05 m) canvas floor in a solid color or with logo/sponsor graphics]

*or*

[Sheet metal flooring, painted to match scoreboard color – add 100 lb (45 kg)]

5. [Penalty indicator arrows; 3" (76 mm) high]

2.04 SCORING CONSOLE

A. Console is an All Sport® 5000 controller
B. Scores multiple sports using changeable keyboard inserts

C. Controls multiple scoreboards, scoring modules and goal lights, including other All Sport 5000 controlled displays currently owned by customer
D. Recalls clock, score, and period information if power is lost

E. Runs Time of Day and Segment Timer modes

F. Console includes:

1. Rugged aluminum enclosure to house electronics   

2. Sealed membrane water-resistant keyboard   

3. 32-character backlit LCD to verify entries and recall information currently displayed 

4. Power cord that plugs into a standard grounded outlet; 6 watts max

5. Control cable to connect to the control receptacle junction box (wired system only)
6. Hand-held switch for main clock start/stop and horn

7. Soft-sided carrying case

G. Accessory Equipment 
(SELECT ONE ACCESSORY IN BRACKETS PER NUMBER OR DELETE ENTIRE NUMBER)
1. [2.4 GHz spread spectrum radio system with frequency hopping technology and 64 non-interfering channels; system includes a transmitter installed inside the console and a receiver installed inside the scoreboard(s)]

2. [Hard carrying case]

Part 3 EXECUTION

3.01 EXAMINATION

A. Verify that mounting surface is ready to receive scoreboard. Verify that placement of conduit and junction boxes are as specified and indicated in plans and shop drawings.
3.02 INSTALLATION

A. Power conduit, power conductors, and signal conduit to within 3' (0.9 m) of displays will be installed by Customer. Control wiring and labor to install will be the responsibility of the contractor assigned to the display equipment. Power conduit, signal conduit, and all electrical connections from within 3' (0.9 m) of displays will be the responsibility of the contractor assigned to the display equipment. 

B. Mount scoreboards and interior displays in location detailed and in accordance with manufacturer's instructions. Unit to be plumb and level.
3.03 installation—control center

A. Provide boxes, cover plates and jacks as required to meet control specification requirements. Control cables to control panels shall be concealed.

B. Test the operation of the scoreboard, controller and all control jacks; leave control unit in carrying case and other loose items with owner's designated representative. 

C. Conduct operator training on the scoreboard/controller operation.
END OF SECTION

PAGE  
SL-05130  030122
H-2105
11 66 43-3

